New World Industries Inc.

# User Setup and Configuration Manual

# CA-101, and CA-102 Proportional Valve Controllers

# Overview

This manual covers installation, configuration and trouble-shooting information for New World Industries CA-101 and CA-102 series proportional hydraulic valve controllers made after December 2008.  For information regarding prior versions, please contact NWI for assistance or look on our website www.nwi-online.com to download a manual.

The CA-101 and CA-102 series proportional hydraulic valve controllers are designed to operate with a wide range of hydraulic valve coils up to 2.5 Amps.  The amplifiers are equipped to accept either 0 – 5 VDC, 0 – 10 VDC and 4 – 20 mA current source as control inputs.  These amplifiers include provisions for joystick zero centering, dead-band and dithering.

# Specifications

Input voltage:  12 – 24 VDC

Output voltage: 12 – 24 VDC depending upon coil resistance

Current control method: High-side PWM control @ 19 kHz with current sensing

Temperature rating: -40C to +125C

Enclosure type: Environmentally Sealed IP67 rated with IP69 connections

Output Channels:

CA-101: 1-channel; 2 valve coils in opposition (providing directional control)

CA-102: 2-channel; 4 valve coils in opposition (providing directional control)

General purpose outputs: 2 @ 2.5Amps each.

General purpose inputs: 4 at rated input voltage.

Input voltage range: 0 – 5 VDC, 0 – 10 VDC, 4 – 20 mA current source

Zero adjustment: 0 – 100 % (Zero may be set anywhere across entire range)

Dithering: 90 – 200 Hz. frequency, up to 2% of maximum current.

# Electrical Connections

Error! No topic specified.

The figure above shows the pin arrangement on the CA-101 and CA-102 viewing the connector on the enclosure.  The gray connector is used for low voltage signals and command inputs.  The black connector contains power, ground and high power signals to the valve coils.  DANGER: incorrectly wiring can damage your unit!  Please double check your connections.

GRAY

- Channel 2 command voltage 0 – 5 VDC & 0 – 10 VDC

- Channel 1 command current source; 4 – 20 mA only

- Channel 2 command current source; 4 – 20 mA only

- Control GND; logical reference for pots, joysticks and other control electronics

- RS-232 TX line

- RS-232 RX line

- CANbus LOW

- CANbus HIGH

- USER + 5VDC; Low current voltage source for use with potentiometers; 100mA maximum load.

- Channel 2 command current sink; 4 – 20 mA only

- Channel 1 command current sink; 4 – 20 mA only

- Channel 1 command voltage; 0 – 5 VDC & 0 – 10 VDC

BLACK

- General purpose input #4

- General purpose input #2

- General purpose output #2

- Valve output 2B

- Valve output 1B

- Power; +12VDC  24VDC

- GROUND

- Valve output 1A

- Valve output 2A

- General purpose output #1

- General purpose input #1

- General purpose input #3

# Setup and Configuration

Required tools:

- Laptop or desktop computer with RS-232 serial port or equivalent.  If you computer does not have a serial port, USB to RS-232 converters are available through NWI, Frys Electronics and many other sources.

- Hyperterminal or equivalent serial console connection.  Hyperterminal is included in Microsoft Window XP under “Accessories.”  Microsoft Windows Vista and 7 no longer include Hyperterminal.  Hyperterminal may now be purchased from Hilgraeve Inc.  Other free options include “Tera Term” and “Real Term,” both are available for free download over the internet.

- Gray and Black mating connectors to connect for your valve controller.

- Power source to activate the controller.

Communication:

Communicating to the CA-101/102 controller is by means of a serial RS232 connection (generally COM1 or COM3 on a PC).  The PA-100 program adapter uses an industry standard DB-9 connector. Plug into a laptop or PC and used the DCC-1 cable and plug into a powered controller once the following setup is complete.

The serial communication parameters are as follows:

- Baud Rate (bits per second):	 115200.

- Data Bits:				 8

- Stop Bits:				 1

- Parity:				 None

- Flow Control:			 Xon/Xoff

After connecting to the controller press <space bar> the “Splash Scan” screen will appear.

New World Ind., http://www.nwi-online.com CA-100 thru CA-102 series proportional valve controller PICos18_v3.00;Mar 09 2008 V-TSK:READY D-TSK:STOP ERR: 0x 0 Coil 1A/B RANGE: 0-5 VDC CMD INPUT: 0 ENABLE: Y CMD CURRENT: 0 mA PWM OUTPUT: 0 ticks CMD OUTPUT: 0 mA ACT OUTPUT: 0 mA DITHER ENABLE: N Coil 2A/B RANGE: 0-5 VDC CMD INPUT: 0 ENABLE: Y CMD CURRENT: 0 PWM OUTPUT: 0 ticks CMD OUTPUT: 0 mA ACT OUTPUT: 0 mA DITHER ENABLE: N Press <e> to edit configuration

Fig 2 Typical HyperTerminal menu screen showing Splash Scan

This screen is used to check controller status and for error codes.

- V-TSK indicates VALVE TASKS are either READY or STOPPED

- D-TSK indicates DITHER TASKS are either READY or STOPPED

- ERR is a self-generated error code for diagnostic purposes.

Press <e> to enter configuration

1A/B CONFIGURATION <key>: Increase / Decrease / Modify Settings <i> : Input range :0-5VDC <2,3>: +/- Minimum Current : 0 mA <4,5>: +/- Maximum Current : 2950 mA <6,7>: +/- Zero Setpoint : 0% of Input range <8,9>: +/- Zero Deadband : 0% of Input range <F,f>: +/- Dither Freq: 0 Hz <A,a>: +/- Dither Amplitude: 0% <U,u>: +/- Up-Ramp seconds: 0 mS <D,d>: +/- Dn-Ramp seconds: 0 mS <C,c>: +/- Rated coil current @ rated voltage: 3000 mA <N> : Next Coil-pair <x> : Press <e> to edit configuration

##### Fig 3 Typical HyperTerminal menu screen showing Configuration Screen

Shown in Fig. 3 is a typical HyperTerminal screen Menu showing the default values for each set of coils.  A description of each configuration item is as follows:

- Input Range:  There are 3 possible settings. Choose which input type by pressing the letter “i” and the 3 selections will scroll thru. 0-5VDC, 0-10VDC or 4-20mA can be selected. Default set to 0-5VDC

- Minimum Current: This setting is the minimum PWM output required for correct valve operation to the A and B coils. Press either 2 or 3 to increase or decrease this setting. Default set to 0

- Maximum Current:  This setting is the maximum PWM output required for correct valve operation to the A and B coils. Press either 4 or 5 to increase or decrease this setting. Default set to 2500 mA

- Zero Setpoint: This variable is used to scale either up or down by percentages of the selected Input Range where the PWM signal starts from. This setting is usually determined by the command input device that’s being used. Press either 6 or 7 to raise or lower the percentage. Default set to 0. Warning: When the “Zero Setpoint” is set above 0 and the input device becomes disabled or disconnected it could still be sending a command signal if the system the controller is operating outputs PWM at 0 command voltage. In that case, NWI recommends integrating an “E-stop between the main power source and the controller. Never work on or disconnect the input device with a pressurized manifold!

- Zero Deadband: This is the percentage of the selected Input Range based on the Zero Setpoint and Maximum Current setting that allows an input voltage deadband when tuning the input device to the controller. Press either 8 or 9 to increase or decrease this setting. Default set to 0

- Dither Frequincy: The Dither Frequency is software adjustable from 0 to 250 hz. Press F or f to increase or decrease the dither frequency. Default set to 0

- Dither Amplitude: The Dither Amplitude is software adjustable from 0 to 10% of the duty cycle.  Press A or a to increase or decrease the dither amplitude. Default set to 0

- Up Ramp: This enables a preset ramp up time in mS. How the ramping up works is, once a value is entered, this is based on a full voltage command signal. An example would be, if the controller is set to run on 0-10VDC and ramping is set to 3000mS, if the full 10VDC command signal is given from a 0VDC start, there will be 3 seconds of ramping.

- Down Ramp: This enables a preset ramp down time in mS. How ramping down works is the exact opposite of ramping up. Example would be, when the command voltage is pulled down to 0 from the maximum command voltage, the controller will ramp down in the time determined by the preset valve entered.

- Rated coil current @ rated voltage: This setting should be determined by the valve manufacturers maximum coil rating and set accordingly to insure full operation of the valve coil.  Default is 3000mA

Note: When configuring the CA-101/102 controller, press “N” to continue to the next Valve configuration screen and repeat the above procedure to set up to 4 coils.

Saving Settings: After configuring the selected solenoid/coils, hit the “x” key to save it to the microprocessor. The controller will retain this configuration until repeating the above process changes the values.

COPYRIGHT, NEW WORLD INDUSTRIES INC. 2009

1 Contact NWI for assistance in the use of these features, some applications may require special programming.